# Memo on Matching Goodtime Intervals Between Direct Events and Histograms

May 14, 2026
N. A. Schwadron

We have encountered difficulties in matching goodtime intervals between Direct Event (DE) data and histogram data. The root of the issue is that the goodtime intervals are generated from the histogram products, but the direct event timing does not precisely match that of the histogram data.

For histogram data interval, we use the following:

- start_time

- end_time = start_time + 300 s

The 300 s extension is necessary to ensure that histogram integrations whose effective accumulation windows extend beyond the nominal interval are excluded from the goodtime selection. This approach prevents additional histogram integrations outside the goodtime window from contaminating the selected data set.

Direct Events behave differently. The DE timestamps exhibit a systematic offset of approximately 7–8 s relative to the start of the corresponding goodtime start intervals. This offset arises from onboard processing delays associated with direct-event handling, prioritization and telemetry generation. As a result, DE events are effectively tagged slightly later than the histogram-derived timing reference.

Because of this offset, applying the same end_time logic to DE data can incorrectly eliminate otherwise valid intervals. In practice, even a modest end-time expansion or contraction can remove legitimate DE events near interval boundaries.

Accordingly, the current approach is:

- Histogram selection:
Require both the histogram start and stop times to remain fully contained within the goodtime interval.

- Direct Event selection:
Apply only the start-time constraint without imposing the same strict end-time offset logic used for histograms.

This distinction preserves valid DE intervals while still maintaining strict containment criteria for histogram integrations.