Summary of Automated Goodtimes Selection Code Modifications 
Fatemeh Rahmanifard

The code was modified to improve the identification of valid (good) time intervals in IMAP-Lo data by introducing an additional contamination check based on ram-direction counts in E-steps 6 and 7.
old version: hist_auto_gt_V2.py
New version: hist_auto_gt_V3.py
 This change was reflected in the shell file “runIMAP-fmv1-autogts.sh” to run the new version of

Key Changes

1. Ram Direction (bins 0–19 and 50–59) H counts, exposure time, and rate were defined for E-steps 6 and 7:
- H counts (hist_auto_gt_V3.py, line 350)
d_ram = (

np.sum(cdf['h_counts'][:, 5:7, 0:20], axis=(1, 2)) +

np.sum(cdf['h_counts'][:, 5:7, 50:60], axis=(1, 2))

)

- Exposure times (hist_auto_gt_V3.py, line 343)
exposure_ram67 = 420 * ncycle_ave * 0.5 * 2.0 / 7.0 #only counting exposure times for E-steps 6 and 7

- H rates (hist_auto_gt_V3.py, lines 442-443):
ram_cnts = np.sum(d_ram[i0:i1])

ram_rate = ram_cnts / exposure_ram67

2. High-Energy Ram Veto:
- The ram-direction count rate computed above sets a new criterion was so that intervals with elevated ram-direction counts are rejected as contaminated. The ram-direction condition was integrated into both the acceptance and closing logic (updates are highlighted, hist_auto_gt_V3.py, lines 447 and 466).
        if (antiram_rate < bg_rate_nom) and (ram_rate < bg_rate_ram_nom):

if (begin == 0.0):

begin = met_from_epoch(epoch[i])

#                if (first_begin == 0.0):

#                    first_begin = begin

bg_rate, bg_rate_unc, bg_orate, bg_orate_unc = background_rate_from_epoch(epoch[i])

# actual background estimate for the bg file

sum_bg_cnts = sum_bg_cnts + (bg_rate * exposure)

sum_og_cnts = sum_og_cnts + (bg_orate * exposure)

sum_bg_expo = sum_bg_expo + exposure

# proxy in the antiram hemisphere

sum_bg1_cnts = sum_bg1_cnts + antiram_cnts_sum

sum_og1_cnts = sum_og1_cnts + antiram_cnts_o_sum

sum_bg1_expo = sum_bg1_expo + exposure_sum

if (antiram_rate >= bg_rate_nom) or (ram_rate >= bg_rate_ram_nom):

if (begin > 0.0):

end = met_from_epoch(epoch[i-1])

#                last_end = end

print_lines(fgt, fcn, date1, begin, end, sum_bg1_cnts, sum_og1_cnts, sum_bg1_expo,  bg_rate_nom, pivot, pivotp)

print_lines_tof_ideas(fideas, date1, begin, end, ideas_header)

begin = 0.0

end = 0.0

3. Configurable Parameter:
- Two new input parameters were introduced for 90 and off-90 (pivot angle) cut rates and were added as a command-line argument to avoid hardcoding and allow flexibility (hist_auto_gt_V3.py, lines 215-225).
    parser.add_argument('-tram90', '--tcutram90',

help='the cut rate for 90 deg PPM ram direction Esteps 6 and 7 (e.g., 0.0028 * 2.0) ',

dest='tcutram90',

type=float,

default=0.0056)

parser.add_argument('-tramoff', '--tcutramOff',

help='the cut rate for PPM angles other than 90 deg ram direction Esteps 6 and 7 (e.g., 0.0035 * 2.0) ',

dest='tcutramOff',

type=float,

default=0.007)

- These parameters were used to set nominal background rates used in the high energy ram veto condition (hist_auto_gt_V3.py, line 328 and 331).
if 88.0 < pivot < 92.0:

bg_rate_nom = args.tcut90

bg_rate_ram_nom = args.tcutram90

else:

bg_rate_nom = args.tcutOff

bg_rate_ram_nom = args.tcutramOff

- These parameters are added to “runIMAP-fmv1-autogts.sh” command line to run hist_auto_gt_V3.py code (updates highlighted, runIMAP-fmv1-autogts.sh, line 31):

$python $pydir/hist_auto_gt_V3.py -f $file -e $file_de -k $file_hk -n 1 -a 7 -d 100 -t90 0.007 -toff 0.00875 -tram90 0.014 -tramoff 0.0175

Conclusion

The added ram-direction veto specifically removes periods with enhanced high-energy flux, leading to cleaner data and more reliable background characterization. The plot below shows the new (including ram-direction veto) vs. old goodtimes for the last 20 days of IMAP-lo H data.